Request Title: Perovskites patterning by photolithography and dry etching.

Requestor: Quentin Thiburce 					Date: 3/30/2021

Non-SNF staff, please complete the following:

Badger ID: thiburce

Company name/PI: Prof. Alberto Salleo

Send completed form to: snf-promcommittee@lists.stanford.edu

- The chemical or material. We will be working with perovskite thin films derived from precursors such as lead iodide and cesium iodide. The CAS numbers for lead iodide and cesium iodide are, respectively: 10101-63-0 and 7789-17-5. I am also attaching the corresponding SDS for both lead iodide and cesium iodide as well as the Congreve Lab SOP for these lead-based nanoparticles.

- Vendor/manufacturer info: We acquire the precursors from the following vendor:

MilliporeSigma:

3050 Spruce St., St. Louis, MO, 63103

1-800-521-8956

https://www.sigmaaldrich.com/united-states.html

- Reason for request: High-resolution patterning of perovskite LEDs which cannot be achieved without the use of photolithography.

- Process Flow: A perovskite thin-film will be deposited on the substrate in the Congreve lab. The perovskite layer will be coated with a Ge hard mask in the AJA evaporator or Lesker sputterer. Alternatively, a Si hard mask can be coated in the HDPCVD system. Standard positive photoresist will then be coated, exposed and developed on top of the perovskite/hard mask stack in a separate facility. Selective dry etching of the hard mask in areas not protected by photoresist will be performed with a CF4 plasma in the PT-OX etcher. The exposed part of the perovskite film with then be etched using Ar ion beam etching in a separate facility. Finally, the Ge hard mask will be removed from the top of the patterned perovskite film with XeF2 vapor in the Xactix chamber.

- Amount and form: Solid perovskite thin-films coated on small substrates (glass slides/silicon pieces) or 4” wafers.

- Storage: No chemical/material will be stored at SNF.

- Disposal: No chemical/material waste will be created.

To be completed by the PROM COMMITTEE:

Special handling requirements:

On-going approval or staff interaction requirements:

Approved By/Date: