Request Title: Request for SnTe target in Lesker Sputter

Requestor: NGUYEN, Ngoc Minh Tri (Tri)					Date: 10/25/2021

Non-SNF staff, please complete the following:

Badger ID: tring@stanford.edu

Company name/PI: Kunal Mukherjee

Send completed form to: snf-promcommittee@lists.stanford.edu

- The chemical or material. Please provide all common names, trade names, and CAS numbers where appropriate. Include an MSDS, if available; or provide the reason, if not. Make sure to include information for any new secondary chemicals (such as a developer for a new resist). Read the MSDSs as well as the Stanford Chemical Storage Groups to determine the Storage Group Identifier and Main Hazard Class of your chemical/material.

Name: Tin Telluride (SnTe)

CAS Number: 12040-02-7

Storage Group: G

Hazard Class: Toxic substances

Attached MSDS and vapor pressure curves of SnTe, Sn and Te as a function of temperature.

Note:

- The materials are toxic and thus they will be handled only by trained staff members.

- The staff members will need to take proper safety precautions when handling the parts of the chamber cleaning as well as the targets.

- Vendor/manufacturer info: address and phone number, website URL.

Vendor: ACI Alloys

Address: 1458 Seareel Place San Jose, CA 95131

Phone: (408) 259-7337

Website URL: https://www.acialloys.com

- Reason for request: Please give serious thought to this. If you have any process information (application notes from the vendor, protocol from another lab, experimental methods section of an article), please include it, preferably as attachments. Ask yourself these questions: Is this the latest procedure? Are there newer/safer alternatives that will also work for my project? Will any of the current SNF approved chemicals and materials work for me?

Need a source to grow SnTe polycrystals as part of the experiments to understand defect tolerance in IV-VI semiconductors.

- Process Flow: Please provide a detailed process flow description on how and where you proposed to use this chemical. This should include all Lab equipment to be used for processing your wafers once your new chemical or material has been used (even if your new material is a film that is removed, it may still pose potential contamination concerns.) Make sure to include wet benches. Please note that f the chemical/material is to be used in any the "clean" equipment, purity specifications will be needed. This is most important for chemicals/material that are not normally used for VLSI device fabrication. To be allowed into a "clean" tool, the material should MOS grade or better.

- Obtain and clean Si wafer.

- Coat the chamber with Ti before and after each growth run to: a) prevent cross-contaminations for the next users and b) minimize the chances of SnTe dust forming during procedures to clean the chamber (and minimize risk to staff). Ti deposition is needed because SnTe has a low meting point.

- SnTe target will be installed and used in Lesker Sputter tool according to the usual sputtering procedure.

- Amount and form. How much will you bring in? Is it solid, powder or liquid? (Note: as a general rule, powders are not permitted in the cleanroom.) Do you need to mix it to use it?

SnTe 99.999% sputtering targets – 2.00 in. diameter x 0.125 in. thick

Bonding info: elastomer or indium bonded to copper backing plate – 2.00 in. diameter x 0.092 in. thick

- Storage: Will you be storing your chemical/material at SNF? If so, please note any potential reactivity’s (this should be on the MSDS). Storage groups A, B, D and L are stored in the yellow solvent cabinet in the furnace support area, while storage groups C, E, F and G are stored on top of one of the Pass-through Carts. Ensure your chemical container or material is properly labeled. If there is no available room, it must be stored by in the bulk storage area. You will then need to obtain it from receiving area personnel each time you want to use it and return it to them when you are finished using it (or each time you leave the lab). Note that there is no storage of chemicals/materials in the processing lab or at any wet bench.

The materials will be stored in SNF Ex-Fab Paul G Allen 155 Mavericks.

- Disposal: How will you dispose of any waste or excess chemical or material? In your discussions with experts and vendors, try to determine the best way to dispose of your spent chemicals and by-products. Please refer to the SNF Lab members Safety Manual for the different methods of waste disposal that are available in the lab.

Target will not be disposed until the material is fully used up. If it needs to be disposed, regular IV-VI disposal procedures will be followed.

To be completed by the PROM COMMITTEE:

Special handling requirements:

On-going approval or staff interaction requirements:

Approved By/Date: