Request Title:  Diazonium Compounds for Carbon Nanotube Surface Functionalization

Requestor: 	Nate Safron					Date:  11/21/2021

Non-SNF staff, please complete the following:

Badger ID:	nsafron@tsmc.com

Company name/PI: TSMC

Send completed form to: snf-promcommittee@lists.stanford.edu

- The chemical or material. Please provide all common names, trade names, and CAS numbers where appropriate. Include an MSDS, if available; or provide the reason, if not. Make sure to include information for any new secondary chemicals (such as a developer for a new resist). Read the MSDSs as well as the Stanford Chemical Storage Groups to determine the Storage Group Identifier and Main Hazard Class of your chemical/material.

- Vendor/manufacturer info: address and phone number, website URL.

- Reason for request: Please give serious thought to this. If you have any process information (application notes from the vendor, protocol from another lab, experimental methods section of an article), please include it, preferably as attachments. Ask yourself these questions: Is this the latest procedure? Are there newer/safer alternatives that will also work for my project? Will any of the current SNF approved chemicals and materials work for me?

- Process Flow: Please provide a detailed process flow description on how and where you proposed to use this chemical. This should include all Lab equipment to be used for processing your wafers once your new chemical or material has been used (even if your new material is a film that is removed, it may still pose potential contamination concerns.) Make sure to include wet benches. Please note that f the chemical/material is to be used in any the "clean" equipment, purity specifications will be needed. This is most important for chemicals/material that are not normally used for VLSI device fabrication. To be allowed into a "clean" tool, the material should MOS grade or better.

- Amount and form. How much will you bring in? Is it solid, powder or liquid? (Note: as a general rule, powders are not permitted in the cleanroom.) Do you need to mix it to use it?

- Storage: Will you be storing your chemical/material at SNF? If so, please note any potential reactivity’s (this should be on the MSDS). Storage groups A, B, D and L are stored in the yellow solvent cabinet in the furnace support area, while storage groups C, E, F and G are stored on top of one of the Pass-through Carts. Ensure your chemical container or material is properly labeled. If there is no available room, it must be stored by in the bulk storage area. You will then need to obtain it from receiving area personnel each time you want to use it and return it to them when you are finished using it (or each time you leave the lab). Note that there is no storage of chemicals/materials in the processing lab or at any wet bench.

- Disposal: How will you dispose of any waste or excess chemical or material? In your discussions with experts and vendors, try to determine the best way to dispose of your spent chemicals and by-products. Please refer to the SNF Lab members Safety Manual for the different methods of waste disposal that are available in the lab.

To be completed by the PROM COMMITTEE:

Special handling requirements:

On-going approval or staff interaction requirements:

Approved By/Date:

  - We want to use 3 different diazonium compounds which are:

    - 4-BromobenzenediazoniumTetrafluoroborate CAS Number: 673-40-5

    - 4-MethoxybenzenediazoniumTetrafluoroborate CAS Number: 459-64-3

    - 4-NitrobenzenediazoniumTetrafluoroborate. CAS Number: 456-27-9

These will be used in aqueous solutions with de-ionized water and require no other chemicals. The main hazard classes of these compounds are corrosive and combustible

  - Sigma-Aldrich Inc.

3050 SPRUCE ST

ST. LOUIS MO 63103

Telephone : +1 314 771-5765

www.sigmaaldrich.com/

  - The purpose of these molecules is to make a covalent self-assembled monolayer on carbon nanotubes. There are no available compounds in SNF for this purpose. The procedure is outlined in “Enhanced Chemical Reactivity of Graphene Induced by Mechanical Strain.” The procedure we propose to use is soaking carbon nanotube coated wafers and pieces of wafers in the diazonium aqueous solutions and then rinsing with DI water after the reaction is completed.

  - This process can be performed at the EXFAB fumehood1 in Mavericks. We will supply dedicated beakers to mix the diazonium solution and for DI water to rinse. The diazonium compounds are powders that need to be weighed and mixed with DI water in the fume hood to avoid inhalation or eye exposure. Also >.11 mm nitrile gloves and a lab jacket need to be worn to avoid skin exposure.

    - First, the diazonium powder is weighed on a scale in the fumehood on weighing paper and added to a beaker with a measured amount of water. The scale must be wiped down with isopropanol and cleanroom wipes after use.

    - Then, a carbon nanotube wafer or piece of wafer is added to the solution.

    - After a certain amount of time, the carbon nanotube wafers are washed in a fresh beaker(s) of DI wafer and then dried with N2.

    - Diazonium solution waste and rinse is disposed in a marked diazonium waste bottle.

    - After functionalization, the sample may continue device fabrication through PVD (AJA evaporator or Lesker sputter), lithography (Headway, SVG Coat, Heidelberg, SVG Develop), ALD (Savannah or Fiji3), and/or etching (Oxford RIE) but the functionalization layer is expected to be stable below 300C and present in extremely low quantities (similar to HMDS).

  - 5 – 10 mgs of each diazonium compound will be brought into the EXFAB in its original packaging with an additional official yellow hazardous materials label and barcode. We cannot bring in pre mixed aqueous solution as the diazonium compounds slowly degrade in water.

  - The diazonium compounds are corrosive and combustible, and temperature sensitive. The compounds can be stored in a refrigerator with Storage Group L (preferably in Mavericks). They are combustible and also compatible with organic solvents, similar to Sodium dodecyl sulfate (SDS) which is stored in group L.

  - These aqueous mixtures containing diazonium and rinse water can be temporarily disposed into a marked waste container specific to these materials. This waste can be temporarily stored in Storage Group G, as it will be no longer combustible. This waste will need to be picked up for local collection.