Request Title:  Terecircuits Terefilm® on Finntech Flip Chip Bonder

Requestor: Matthew Robinson				Date:  25 January, 2021

Non-SNF staff, please complete the following:

Badger ID:		mrobinson@terecircuits.com

Company name/PI: Terecircuits Corp/Matthew Robinson

Send completed form to: snf-promcommittee@lists.stanford.edu

- The chemical or material. Please provide all common names, trade names, and CAS numbers where appropriate. Include an MSDS, if available; or provide the reason, if not. Make sure to include information for any new secondary chemicals (such as a developer for a new resist). Read the MSDSs as well as the Stanford Chemical Storage Groups to determine the Storage Group Identifier and Main Hazard Class of your chemical/material.

Poly(1,2,3,4-tetrahydronaphthalene-1,4-diol carbonate); SDS appended to end of this form; we recommend Chemical Storage Group G. It does burn when you hold an open flame to it, but the combustion is not self-sustaining.

- Reason for request: Please give serious thought to this. If you have any process information (application notes from the vendor, protocol from another lab, experimental methods section of an article), please include it, preferably as attachments. Ask yourself these questions: Is this the latest procedure? Are there newer/safer alternatives that will also work for my project? Will any of the current SNF approved chemicals and materials work for me?

This polymer is the core IP and value proposition of our company. No other chemical would suffice, or we would not have a value proposition. Our proprietary process of photochemical degradation will be conducted at our facility on customer equipment. We need the Finetech Lambda equipment at SNF to explore the process space to bond unprocessed silicon chips to fused silica plates using Terefilm as the boding adhesive. Once we know the bonding process space, we will contract for custom equipment to be built for our customers.

- Process Flow: Please provide a detailed process flow description on how and where you proposed to use this chemical. This should include all Lab equipment to be used for processing your wafers once your new chemical or material has been used (even if your new material is a film that is removed, it may still pose potential contamination concerns.) Make sure to include wet benches. Please note that f the chemical/material is to be used in any the "clean" equipment, purity specifications will be needed. This is most important for chemicals/material that are not normally used for VLSI device fabrication. To be allowed into a "clean" tool, the material should MOS grade or better.

See “Pre-press Schematic for SNF” process flow document (separate attachment). For this request I am proposing only Step 3 be done at SNF. I will submit a PROM later to do Steps 1 & 2 at SNF, although SMF might be a more appropriate location.

We are looking to investigate the whole parameter space between:

Room Temperature to 150C

Lowest pressure setpoint to 100kg of force or the maximum setpoint, whichever is lower

Contact Times from 10 seconds to 5 minutes (longer times at lower temperatures and pressure)

Our current baseline process is 140C for 10 seconds at 10N of force ramping up to temperature and down from temperature as fast as achievable.

- Amount and form. How much will you bring in? Is it solid, powder or liquid? (Note: as a general rule, powders are not permitted in the cleanroom.) Do you need to mix it to use it?

10 samples per visit. Each sample will be a 1-2um thick x 2”x2” area dry film on fused silica. This equates to 2.5 mg per sample of Terefilm. No processing will occur at SNF without a further PROM request.

- Storage: Will you be storing your chemical/material at SNF? If so, please note any potential reactivity’s (this should be on the MSDS). Storage groups A, B, D and L are stored in the yellow solvent cabinet in the furnace support area, while storage groups C, E, F and G are stored on top of one of the Pass-through Carts. Ensure your chemical container or material is properly labeled. If there is no available room, it must be stored by in the bulk storage area. You will then need to obtain it from receiving area personnel each time you want to use it and return it to them when you are finished using it (or each time you leave the lab). Note that there is no storage of chemicals/materials in the processing lab or at any wet bench.

No

- Disposal: How will you dispose of any waste or excess chemical or material? In your discussions with experts and vendors, try to determine the best way to dispose of your spent chemicals and by-products. Please refer to the SNF Lab members Safety Manual for the different methods of waste disposal that are available in the lab.

Disposal will be done at own own facility except if samples drop and break. In that case I suggest disposing in regular uncontaminated sharp container unless the committee suggests otherwise.

- Supplemental: Swaroop Kommera raised two additional concerns that I will address here:

  - What decomposition product evolves from the polymer at those temperatures if any.

  - What I intend to do to protect the instrument from the sample flowing to its parts.

The TGA of this material is pasted below.

By 120C, 1.1% of the mass is lost. We have confirmed this to be residual solvent (PGMEA). This amount is equivalent to 27ug or 27 uL. By 150C we lose an additional 0.6% of the mass. This is the decomposition of the polymer into14 nanomoles of CO2 and 14 nanomoles of naphthalene per sample. Both will be gasses at that temperature, although the amount is minuscule.

After reviewing the second concern with Leigh Jackson, we have zero concern that the material can flow off the substrate. First of all the sample is a glassy solid up to 140C, so below 140C there is zero tack. Above 140C it undergoes a glass transition to a rubber state. Even if it would be a pure liquid and the substrate would be vertical or upside down, 1um films do not leave their substrates – gravity is a far weaker force than the capillary forces keeping the 1um material on the substrate.

To be completed by the PROM COMMITTEE:

Special handling requirements:

On-going approval or staff interaction requirements:

Approved By/Date:

Terecircuits Corporation

1805 Little Orchard St, #2131, San Jose CA 95125	Effective Date: 15 July 2018

(650) 687-7095‬

www.terecircuits.com

## SAFETY DATA SHEET

SECTION 1 – PRODUCT AND COMPANY IDENTIFICATION

1.1	Product identifiers

Product name: Poly(1,2,3,4-tetrahydronaphthalene-1,4-diol carbonate)

Product number: n/a

CAS-No.: not assigned

1.2	Relevant identified uses of the substance or mixture and uses advised against

Identified uses:	microelectronic system integration processing

1.3	Details of the supplier of the safety data sheet

Company:	Terecircuits Corporation

1805 Little Orchard St, #131

San Jose CA 95125

Tel.: +1 650-687-7095

1.4	Emergency Telephone Numbers

+1 650-687-7095

SECTION 2 - HAZARDS IDENTIFICATION

2.1	Classification

Not a hazardous substance or mixture.

2.2	GHS Label elements, including precautionary statements

Not a hazardous substance or mixture.

2.3	Hazards not otherwise classified (HNOC) or not covered by GHS – none

# SECTION 3 - COMPOSITION / INFORMATION ON INGREDIENTS

# 3.1	Substances

# Formula:	C11H10O3

# CAS-No.:	none assigned

SECTION 4 - FIRST AID MEASURES

4.1	Description of first aid measures

If inhaled

If breathing, move person into fresh air. If not breathing, give artificial respiration.

In case of skin contact

Wash off with soap and plenty of water.

In case of eye contact

Flush eyes with water as a precaution.

If swallowed

Rinse mouth with water and get medical attention.

4.2	Most important symptoms and effects, both acute and delayed

No data available. See also section 11.

4.3	Indication of any immediate medical attention and special treatment needed

No data available

# SECTION 5 - FIRE FIGHTING MEASURES

# 5.1	Extinguishing media

# Suitable extinguishing media

# Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

# 5.2	Special hazards arising from the substance or mixture

# Carbon oxides and other potentially toxic or irritating fumes may be given off during burning or thermal decomposition.

# 5.3	Advice for firefighters

# Wear self-contained breathing apparatus for firefighting.

# 5.4	Further information

# No data available

# SECTION 6 - ACCIDENTAL RELEASE MEASURES

# 6.1	Personal precautions, protective equipment and emergency procedures

# Avoid dust formation. Avoid breathing vapours, mist or gas. For personal protection see section 8.

# 6.2	Environmental precautions

# No special environmental precautions required.

# 6.3	Methods and materials for containment and cleaning up

# Sweep up and shovel. Keep in suitable, closed containers for disposal.

# 6.4	Reference to other sections

# For disposal see section 13.

# SECTION 7 - HANDLING AND STORAGE

# 7.1	Precautions for safe handling

# Further processing of solid materials may result in the formation of combustible dusts. The potential for combustible dust formation should be taken into consideration before additional processing occurs.

# Provide appropriate exhaust ventilation at places where dust is formed.

# 7.2	Conditions for safe storage, including any incompatibilities

# Keep container tightly closed in a dry and well-ventilated place.

# Recommended storage temperature 2 - 8 °C

# Storage class (TRGS 510): Non Combustible Solids

# 7.3	Specific end use(s)

# Apart from the uses mentioned in section 1.2, no other specific uses are stipulated

# SECTION 8 - EXPOSURE CONTROLS/PERSONAL PROTECTION

## 8.1	Control parameters

## Components with workplace control parameters

## Contains no substances with occupational exposure limit values.

## 8.2	Exposure controls

## Appropriate engineering controls

## General industrial hygiene practice.

## Personal protective equipment

## Eye/face protection

## Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

## Skin protection

## Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands.

## Body Protection

## When used according to section 1.2, customary laboratory clothing and protection is recommended.

## Respiratory protection

## Respiratory protection is not required. Where protection from nuisance levels of dusts are desired, use type N95 (US) or type P1 (EN 143) dust masks. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

## Control of environmental exposure

## No special environmental precautions required.

## SECTION 9 - PHYSICAL AND CHEMICAL PROPERTIES

## 9.1.	Information on basic physical and chemical properties

## a) Appearance				Solid white or off-white powder

## b) Odour					None

## c) Odour Threshold			n/a (not applicable)

## d) pH					n/a

## e) Melting point/freezing point		Decomposes before melting

## f) Boiling point and boiling range	Decomposes before boiling

## g) Flash point				No data available

## h) Evaporation rate			n/a

## i) Flammability (solid, gas)		HMIS flammability rating 0

## j) Upper/lower flammability or		n/a

## explosive limits

## k) Vapour pressure			Zero at 300K

## l) Vapour density			n/a

## m) Density relative to water		About 1.3

## n) Water solubility			Insoluble

## o) Partition coefficient: n- octanol/water	n/a

## p) Auto-ignition temperature		No data available

## q) Decomposition temperature		150 - 180C

## r) Viscosity				n/a

## s) Explosive properties			None

## t) Oxidizing properties			None

## 9.2	Other safety information

## No data available

SECTION 10 - STABILITY AND REACTIVITY

10.1	Reactivity

Stable except in the presence of strong oxidizing agents or heat.

10.2	Chemical stability

Stable under recommended storage conditions.

10.3	Possibility of hazardous reactions

None other than noted in section 10.1

10.4	Conditions to avoid

Temperatures above 150C

10.5	Incompatible materials

Strong oxidizing agents

10.6	Hazardous decomposition products

Decomposition produces carbon dioxide and naphthalene.

When burned may form toxic levels of carbon monoxide, carbon dioxide, and other unidentified organic molecules.

# SECTION 11 - TOXICOLOGICAL INFORMATION

# 11.1 Information on toxicological effects

# Acute toxicity

# No data available. See 11.2.

# Inhalation: No data available

# Dermal: No data available

# Skin corrosion/irritation

# No data available. See 11.2.

# Serious eye damage/eye irritation

# No data available.

# Respiratory or skin sensitisation

# No data available

# Germ cell mutagenicity

# No data available

# Carcinogenicity

IARC: No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

ACGIH:	No component of this product present at levels greater than or equal to 0.1% is identified as a carcinogen or potential carcinogen by ACGIH.

NTP: No component of this product present at levels greater than or equal to 0.1% is identified as a known or anticipated carcinogen by NTP.

OSHA: No component of this product present at levels greater than or equal to 0.1% is identified as a carcinogen or potential carcinogen by OSHA.

Reproductive toxicity

No data available

Specific target organ toxicity - single exposure

No data available. See section 11.2.

Specific target organ toxicity - repeated exposure

No data available. See section 11.2.

Aspiration hazard

No data available

11.2	Additional Information

The chemical, physical, and toxicological properties have not been thoroughly investigated. To the best of our knowledge, there are no unusual toxic properties.  Customary laboratory procedures for handling newly synthesized polymers are recommended. See section 16.

# SECTION 12 - ECOLOGICAL INFORMATION

# No information available.

# SECTION 13 - DISPOSAL CONSIDERATIONS

# 13.1	Waste treatment methods

# Product

# Offer surplus and non-recyclable solutions to a licensed disposal company.

# Contaminated packaging

# Dispose of as unused product.

# SECTION 14 - TRANSPORTATION INFORMATION

DOT (US)

Not dangerous goods

IMDG

Not dangerous goods

IATA

Not dangerous goods

SECTION 15 - REGULATORY INFORMATION

SARA LISTED: No

TSCA Inventory Item: No

California Prop. 65 Components

This product does not contain any chemicals known to State of California to cause cancer, birth defects, or any other reproductive harm.

SECTION 16 - OTHER INFORMATION

HMIS rating

Health hazard: 0

Flammability: 0

Physical Hazard 0

NFPA Rating

Health hazard: 0

Fire Hazard: 0

Reactivity Hazard: 0

Toxicological data for similar material. Experiments reported in the Chemical and Pharmaceutical Bulletin, vol. 31(4), pp 1400–1408 for the chemically closely related material poly(propylene carbonate) showed no degradation or toxicity during in vivo exposure of rats for 2 months. Experiments reported in Macromolecular Research, vol. 16(5), pp 473-480 (2008) showed no inflammatory response after 12 weeks of subcutaneous implantation in mice beyond expectation for normal surgical implants.

The material is structurally related to poly(propylene carbonate), for which safety data sheets are available from vendors.  The recommendations herein are consistent with recommendations for this material and others which are part of this class of polymers, for example poly(cyclohexene carbonate).

It is believed that the herein contained information is accurate.  The information is provided for the purpose of hazard communication independent of any sale of the product. There is no claim made regarding an implied warranty, express warranty, and fitness for a particular purpose or merchantability by the information herein contained or regarding the product.

User should consult his legal advisor or the appropriate government agency to establish applicability or effect of any regulation or law regarding the product. We do not undertake to provide advice on these matters.