Request Title:  Perminex 1000 Series

Requestor: Benjamin Micklavzina						Date:  12/7/2021

Non-SNF staff, please complete the following:

Badger ID: bmicklavzina@chronushealth.com

Company name/PI: Chronus Health Inc.

Send completed form to: snf-promcommittee@lists.stanford.edu

- The chemical or material: Perminex 1000 Series Resists. Product numbers  Y514020, Y514046, Y514058, Y514062 from Kayaku Advanced Materials. MSDS included with email, but linked here MSDS_PermiNex™_1000_Series_Resists_-_US (ufl.edu). Classified L as a flammable substance.

It uses PN1000 developer. Product number Y056075 from Kayaku Advanced Materials. MSDS included with email, but linked here SDS_PN_1000_Developer_-_US (uwaterloo.ca). Classified L as a flammable substance.

- Vendor/manufacturer info: Kayaku Advanced Materials.

200 Flanders Road

Westborough, MA 01581 USA

Tel: 617-965-5511

PermiNex Wafer Bonding Adhesives | Kayaku Advanced Materials (kayakuam.com)address and phone number, website URL.

- Reason for request: Perminex is used for adhesive bonding two wafers, and the nearest photoresist to it that could work (SU8) bonds poorly with glass. We cannot use silicon due to its capacitive properties. The data sheet included in the email walks through an example process on a 6” wafer.

- Process Flow:

  - Glass wafers are cleaned with Piranha solution at a wet bench.

  - Wafers are spin coated (likely at the headway 2) and edge bead removal is performed

  - Soft bake on hotplates

  - Alignment and exposure are done on a karl suss ma6

  - Post exposure bake on hotplates

  - Development at a wet bench in pn-1000

  - Adhesive bonding. Could be done in EV wafer bonder if allowed, otherwise done in a heated press offsite.

- Amount and form. 500 mL of liquid photoresist in a glass bottle.

- Storage: Storage in group L

- Disposal: Storage of liquid waste will be done in a carboy container.

To be completed by the PROM COMMITTEE:

Special handling requirements:

On-going approval or staff interaction requirements:

Approved By/Date: