Request Title:  Process Development of ZPI-2201-CH Colorless Polyimide

Requestor: 	Amit Gupta						Date: August 26, 2021

Non-SNF staff, please complete the following:

Badger ID:	agupta66@gmail.com

Company name/PI: Royole Corporation

Send completed form to: snf-promcommittee@lists.stanford.edu

- The chemical or material. Please provide all common names, trade names, and CAS numbers where appropriate. Include an MSDS, if available; or provide the reason, if not. Make sure to include information for any new secondary chemicals (such as a developer for a new resist). Read the MSDSs as well as the Stanford Chemical Storage Groups to determine the Storage Group Identifier and Main Hazard Class of your chemical/material.

The name of the product is ZPI-2201-CH.  The chemical is also called Z2 CH varnish.  This varnish uses cyclohexanone as the solvent.  Below are the constituents of the chemical and their corresponding CAS numbers.

| Component | CAS # | Amount |
| --- | --- | --- |
| Cyclohexanone | 108-94-1 | 85-90% |
| Polyimide | N/A | 10-15% |

Please find attached the MSDS for the chemical in the same folder as this document.

- Vendor/manufacturer info: address and phone number, website URL.

Zymergen, Inc.

5980 Horton Street #105

Emeryville CA 94608

Main: +1 (415) 801-8073 (Available M-F, 8 am - 5 pm PT)

E-mail: info@zymergen.com

Emergency telephone number Chemtrec (24-hour availability):

+1 (800) 424-9300 (USA and Canada)

+1 (703) 527-3887 (International; collect calls accepted)

- Reason for request: Please give serious thought to this. If you have any process information (application notes from the vendor, protocol from another lab, experimental methods section of an article), please include it, preferably as attachments. Ask yourself these questions: Is this the latest procedure? Are there newer/safer alternatives that will also work for my project? Will any of the current SNF approved chemicals and materials work for me?

We would like to test the planarization capability of this varnish in our process.  The varnish is a colorless polyimide with a low curing temperature of around 230C and can be used to planarize structures with height of 5um-6um.  We will most likely perform patterning of the material using PT-Ox or Oxford-RIE to perform the dry etching of the polymer.  We would also determine the surface roughness of the varnish after curing of the material.

- Process Flow: Please provide a detailed process flow description on how and where you proposed to use this chemical. This should include all Lab equipment to be used for processing your wafers once your new chemical or material has been used (even if your new material is a film that is removed, it may still pose potential contamination concerns.) Make sure to include wet benches. Please note that f the chemical/material is to be used in any the "clean" equipment, purity specifications will be needed. This is most important for chemicals/material that are not normally used for VLSI device fabrication. To be allowed into a "clean" tool, the material should MOS grade or better.

Below are the proposed processing steps:

- Spin coat the ZPI-CH varnish on headway.

- Soft-bake using hotplates at 70-110C.

- Cure the resin using either TEK Oven or the BlueM Oven at up to 230C for at least 30 min

- Measure step height using Profilometer and surface roughness using afm2.

- Spin coat SPR220-3 resist using svgcoat1/svgcoat2.

- Expose using contact aligner karlsuss/karlsuss2.

- Develop resist using svgdev/svgdev2.

- Perform dry etching using PT-Ox or Oxford-RIE using gases oxygen and CHF3.

We will be using our own chemical container to store and handle the varnish.

All excess solvent and varnish will be poured into the solvent waste carboys and wipes, foils, waste swabs etc. will be placed into zip-locks and disposed-off into the solvent trash.

- Amount and form. How much will you bring in? Is it solid, powder or liquid? (Note: as a general rule, powders are not permitted in the cleanroom.) Do you need to mix it to use it?

We will store a bottle of ~300mL of the liquid varnish at room temperature at SNF.

- Storage: Will you be storing your chemical/material at SNF? If so, please note any potential reactivity’s (this should be on the MSDS). Storage groups A, B, D and L are stored in the yellow solvent cabinet in the furnace support area, while storage groups C, E, F and G are stored on top of one of the Pass-through Carts. Ensure your chemical container or material is properly labeled. If there is no available room, it must be stored by in the bulk storage area. You will then need to obtain it from receiving area personnel each time you want to use it and return it to them when you are finished using it (or each time you leave the lab). Note that there is no storage of chemicals/materials in the processing lab or at any wet bench.

We propose to store the colorless polyimide varnish in the pass-through area of SNF where chemicals are room temperature.

- Disposal: How will you dispose of any waste or excess chemical or material? In your discussions with experts and vendors, try to determine the best way to dispose of your spent chemicals and by-products. Please refer to the SNF Lab members Safety Manual for the different methods of waste disposal that are available in the lab.

We will dispose-off excess varnish into the solvent waste carboys or seal in plastic zip lock bags and dispose-off in the solvent waste receptacles.

To be completed by the PROM COMMITTEE:

Special handling requirements:

On-going approval or staff interaction requirements:

Approved By/Date: