Request Title:  	Request to use color resist SW-7001

Requestor: 		Tracy Lu			Date:  Jul. 12th , 2020

Non-SNF staff, please complete the following:

Badger ID:	tracylu@royole.com

Company name/PI: Royole Corporation

Send completed form to: snf-promcommittee@lists.stanford.edu

- The chemical or material. Please provide all common names, trade names, and CAS numbers where appropriate. Include an MSDS, if available; or provide the reason, if not. Make sure to include information for any new secondary chemicals (such as a developer for a new resist). Read the MSDSs as well as the Stanford Chemical Storage Groups to determine the Storage Group Identifier and Main Hazard Class of your chemical/material.

SW-7001 is a color resist from Fujifilm. Below are the constituents of the chemical and their corresponding CAS numbers.

SW-7001 hazardous ingredients:

Please find attached the MSDS for the chemical in the same folder as this document.

- Vendor/manufacturer info: address and phone number, website URL.

FUJIFILM Electronic Materials U.S.A., Inc.

80 Circuit Road, North Kingstown, RI 02852.

Telephone: 1-800-553-6546.

https://www.fujifilm.com/us/en/business/semiconductor-materials/image-sensor-color-mosaic/transparent/applications

- Reason for request: Please give serious thought to this. If you have any process information (application notes from the vendor, protocol from another lab, experimental methods section of an article), please include it, preferably as attachments. Ask yourself these questions: Is this the latest procedure? Are there newer/safer alternatives that will also work for my project? Will any of the current SNF approved chemicals and materials work for me?

SW-7001 is a transparent material designed for color reproduction by employing an exacting design of primary color(RGB) transmittance with optimized color separation. The material is clear so it can serve as a planarization layer.

Since we’re developing advanced display technology with QDPR and color filter, we need transparent material to provide good adhesion and encapsulation for our devices. Based on our evaluation of CT-3050(previously approved by PROM committee in 2020), it can’t provide a good coverage on the edge of our patterns. So Fujifilm vendor recommended this SW-7001 as a substitute, which can perform a thicker clear film.

- Process Flow: Please provide a detailed process flow description on how and where you proposed to use this chemical. This should include all Lab equipment to be used for processing your wafers once your new chemical or material has been used (even if your new material is a film that is removed, it may still pose potential contamination concerns.) Make sure to include wet benches. Please note that f the chemical/material is to be used in any the "clean" equipment, purity specifications will be needed. This is most important for chemicals/material that are not normally used for VLSI device fabrication. To be allowed into a "clean" tool, the material should MOS grade or better.

Below are the proposed processing steps:

- Spin coating SW-7001 on Headway.

- Soft-bake using hotplate at 100C for 2mins.

- Expose using Karlsuss Contact Aligner

- Develop using CD2060 and rinse with DI water on the wbmiscres/Headway2 bench. CD2060 is a diluted TMAH solution for the color resist, which has been applied by other lab users and approved PROM committee before

- Hard-bake using hotplate at 220C for 5mins.

All excess solvent and resist will be poured into the solvent waste carboys and wipes, foils, waste swabs etc. will be placed into zip-locks and disposed-off into the solvent trash can.

- Amount and form. How much will you bring in? Is it solid, powder or liquid? (Note: as a general rule, powders are not permitted in the cleanroom.) Do you need to mix it to use it?

We will store one bottle of 100mL of SW-7001 in the refrigerator at SNF, together with other approved color resists.  One bottle of 4L CD2060 developer will be stored at room temperature in the metal flammables cabinet.

- Storage: Will you be storing your chemical/material at SNF? If so, please note any potential reactivity’s (this should be on the MSDS). Storage groups A, B, D and L are stored in the yellow solvent cabinet in the furnace support area, while storage groups C, E, F and G are stored on top of one of the Pass-through Carts. Ensure your chemical container or material is properly labeled. If there is no available room, it must be stored by in the bulk storage area. You will then need to obtain it from receiving area personnel each time you want to use it and return it to them when you are finished using it (or each time you leave the lab). Note that there is no storage of chemicals/materials in the processing lab or at any wet bench.

We propose to store the resist in the freezer inside the SNF cleanroom, where all the other resists, spin-on-glass, etc. are stored.

- Disposal: How will you dispose of any waste or excess chemical or material? In your discussions with experts and vendors, try to determine the best way to dispose of your spent chemicals and by-products. Please refer to the SNF Lab members Safety Manual for the different methods of waste disposal that are available in the lab.

We will dispose-off excess resist into the solvent waste carboys or seal in plastic zip lock bags and dispose-off in the solvent waste receptacles.

To be completed by the PROM COMMITTEE:

Special handling requirements:

On-going approval or staff interaction requirements:

Approved By/Date: