Request Title:  	Epoxy Options for Non-galvanic Flip Chip Bonding

Requestor: Rachel Gruenke						Date:  10/09/2019

Non-SNF staff, please complete the following:

Badger ID:	rgruenke

Company name/PI: Amir Safavi-Naeini

Send completed form to: snf-promcommittee@lists.stanford.edu

- The chemical or material. Please provide all common names, trade names, and CAS numbers where appropriate. Include an MSDS, if available; or provide the reason, if not. Make sure to include information for any new secondary chemicals (such as a developer for a new resist). Read the MSDSs as well as the Stanford Chemical Storage Groups to determine the Storage Group Identifier and Main Hazard Class of your chemical/material.

  - Loctite Stycast 2850FT

  - Loctite Catalyst 24LV

  - GE Varnish (VGE-7301)/Ethanol solution (75% by volume VGE-7301, 25% by volume ethanol)

- Vendor/manufacturer info: address and phone number, website URL.

  - Stycast 2085FT

    - Vendor website (Digikey): https://www.digikey.com/catalog/en/partgroup/loctite-stycast-2850ft-and-loctite-stycast-2850kt-catalyst-11-series/87425

    - Manufacturer website (Henkel): https://www.henkel-adhesives.com/us/en/product/potting-compounds/loctite_stycast_2850ft.html

  - Catalyst 24LV

    - Vendor website (Digikey): https://www.digikey.com/catalog/en/partgroup/loctite-stycast-2850ft-and-loctite-stycast-2850kt-catalyst-11-series/87425 (same as above)

    - Manufacturer website (Henkel): https://www.henkel-adhesives.com/us/en/product/encapsulants/loctite_stycast_2850ktcat24lv.html

  - GE Varnish (VGE-7301)/Ethanol

    - Vendor & Manufacturer website (Lakeshore): https://www.lakeshore.com/products/categories/overview/temperature-products/cryogenic-accessories/varnish

- Reason for request: Please give serious thought to this. If you have any process information (application notes from the vendor, protocol from another lab, experimental methods section of an article), please include it, preferably as attachments. Ask yourself these questions: Is this the latest procedure? Are there newer/safer alternatives that will also work for my project? Will any of the current SNF approved chemicals and materials work for me?

I am developing a non-galvanic flip chip bonding method, using epoxy or photoresist as a bonding agent. This process gives us more control in our bonding separation than indium bump-type bonding while keeping our circuit parameters unchanged. The process requires less than a nanoliter drop of epoxy to be applied onto two edges of the top chip surface chip with a needle or pipette. We have tried using SPR 220-7 photoresist, but it is too brittle to be thermal cycled in a dilution refrigerator; our SPR 220-7 between the two chips fractures during a second thermal cycling to 4K, and we need our chips to thermal cycle at minimum 3 times (but it could certainly be more based on experimental needs). To solve this problem, I have thermal cycled two sets of old broken chips (Al on Si): one set bonded with VGE-7301, one set bonded with Stycast 2850 epoxy + catalyst 24LV. Both survived 3 thermal cycles, with no signs of brittleness. Thus, we have the data to demonstrate that we need these agents to allow us to continue with non-galvanic flip chip bonding. We would like to request both VGE-7301 and Stycast+Catalyst because we are not sure if either will effect the quality factors of our fabricated devices, which can only be tested after we completely fabricate good devices of both bonding types in the cleanroom.

- Process Flow: Please provide a detailed process flow description on how and where you proposed to use this chemical. This should include all Lab equipment to be used for processing your wafers once your new chemical or material has been used (even if your new material is a film that is removed, it may still pose potential contamination concerns.) Make sure to include wet benches. Please note that f the chemical/material is to be used in any the "clean" equipment, purity specifications will be needed. This is most important for chemicals/material that are not normally used for VLSI device fabrication. To be allowed into a "clean" tool, the material should MOS grade or better.

Step 1: Enable the Flip Chip Bonder tool (contaminated category) in ExFab. Align our top chip and prepare the recipe

Step 2: Bring bottom chip to the ExFab solvent bench (all category). Place the Al on Si chip with SU-8 spacers (already in the contaminated category) on a piece of aluminum foil in case there is any epoxy runoff from the chip edge.

If VGE-7301:

Step 3: Using a needle and syringe or another approved sharp, apply a nanoliter drop of VGE-7301 and spread along the length of the chip outside of the SU-8 spacer fabricated to separate the chips and keep the glue from the center of the chip. Repeat to place glue on the other length of the chip.

If Stycast 2850:

Step 3: First mix the Stycast 2850 epoxy with the Catalyst 24LV at a 100:8 weight ratio. To mix, we will weigh the stycast and catalyst on a group scale into a plastic disposable mixing cup using a plastic stir bar. Using a needle and syringe or another approved sharp, apply a nanoliter drop of mixed Stycast and spread along the length of the chip outside of the SU-8 spacer fabricated to separate the chips and keep the glue from the center of the chip. Repeat to place glue on the other length of the chip.

Step 4: Bring the bottom chip back to the flip chip bonder and place on bottom vacuum substate holder.

Step 5: Return to the solvent bench and dispose of the glue waste.

Step 6: Align the chips using the flip chip bonder tool, bring the flip chip bonder arm down to bond the chips, enable temperature profile, and complete the non-galvanic bonding. Remove the chip and disable the flip chip bonder tool after the temperature profile completes.

- Amount and form. How much will you bring in? Is it solid, powder or liquid? (Note: as a general rule, powders are not permitted in the cleanroom.) Do you need to mix it to use it?

All three materials are in liquid form. Our VGE-7301/ethanol solution will be mixed before entering the cleanroom. The Stycast 2850 and Catalyst 24LV need to be mixed right before curing. We would like to bring in 30 mL of each material in separate cleaned jars.

- Storage: Will you be storing your chemical/material at SNF? If so, please note any potential reactivity’s (this should be on the MSDS). Storage groups A, B, D and L are stored in the yellow solvent cabinet in the furnace support area, while storage groups C, E, F and G are stored on top of one of the Pass-through Carts. Ensure your chemical container or material is properly labeled. If there is no available room, it must be stored by in the bulk storage area. You will then need to obtain it from receiving area personnel each time you want to use it and return it to them when you are finished using it (or each time you leave the lab). Note that there is no storage of chemicals/materials in the processing lab or at any wet bench.

All three products are in storage group L, as they are solvent based. Thus, we would like to store these with other solvent materials, preferably near the ExFab Solvent Bench where they would ideally be used.

- Disposal: How will you dispose of any waste or excess chemical or material? In your discussions with experts and vendors, try to determine the best way to dispose of your spent chemicals and by-products. Please refer to the SNF Lab members Safety Manual for the different methods of waste disposal that are available in the lab.

All of these products are solvent based, so they will be disposed of with solvent waste. The syringe and needle used for application will be place in a disposable bag and go in the box for solvent contaminated sharps. The foil used under the chip during application that may collect some of the materials will go in a disposable bag and go in solvent waste.

To be completed by the PROM COMMITTEE:

Special handling requirements:

On-going approval or staff interaction requirements:

Approved By/Date: