Request Title:   	Zirconium Standard

Requestor: 	Eva Strzelecka						Date:  2/14/2019

Non-SNF staff, please complete the following:

Badger ID:	evastanfordfab@gmail.com

Company name/PI:  Raxium

Send completed form to: snf-promcommittee@lists.stanford.edu

- The chemical or material. We would like in bring in Zirconium Standard for ICP, which is a solution of Zirconium in Hydrochloric Acid and Water. Other common names are: Zirconium ICP Standard, Zirconium Standard. Standards like this are used in analytical techniques such as ICP-MS. Per manufacturer, the Zirconium is from Zirconium oxychloride octahydrate, and the HCl concentration is 10% in water. The SDS sheet is attached. CAS: 7440-67-7,7647-01-0. As for storage, it should be stored with acids and other corrosives. Per SNF, this material will be considered Semi-Clean B, because it contains Zr. The main hazard is that it is corrosive because it contains hydrochloric acid. The concentration is less (but on same order of magnitude) than the hydrochloric acid approved and commonly used at SNF wet benches. The toxicity of Zirconium itself is low.  Zirconium is also not a new compound at SNF, as it is used in deposition tools.

- Vendor/manufacturer info: Vendor: Fischer Scientific 1-800-766-7000 www.fishersci.com (There are many alternative vendors, such as VWR, Agilent.).

- Reason for request: The reason for this request is to test the effect of Zr surface concentration on our wafers.  The use of a standard was motivated by the fact that it has a known concentration of the metal and common and clean chemicals.  Zr should not pose a great risk to other technologies, as it is already used in semiconductors: Zr in zirconium silicate is used to replace silicon dioxide as a high-k gate dielectric. Other uses of Zr are as a diffusion barrier in metallurgy, and in ceramics. If there is any concern by SNF members, we can use the chemical in our own beakers instead of the shared ones.

- Process Flow: We will treat the wafers in a solution of the Zirconium Standard, diluted in the same concentration of hydrochloric acid as the carrier (10%).  The chemicals will be used in wet benches: Wbflexcorr 1-2 or 3-4.  After a timed soak, the wafers will be rinsed in DI water, blow dried and then sealed in carriers at the vacuum sealer. The analysis of the samples will most likely not be at SNF. This chemical will have a SPEX certiprep certification, per supplier.

- Amount and form. The amount will be a 500ml bottle of liquid. For our use, we will dilute it in a 10% solution of HCl in water.

- Storage: We will be storing the chemical at SNF. It is a corrosive (10% HCl) and should be stored with storage group F (Inorganic acids).  If so, please note any potential reactivity’s (this should be on the MSDS). We will ensure the chemical container is properly labeled.

- Disposal: The chemical will be disposed the same way as hydrochloric acid, aspirated at the wet bench into the AWN system.

To be completed by the PROM COMMITTEE:

Special handling requirements:

On-going approval or staff interaction requirements:

Approved By/Date: