Request Title: Depositions on V-VI Materials in Fiji3

Requestor: 	Molly Andersen				Date:  12/19/2019

Non-SNF staff, please complete the following:

Badger ID:	molly83

Company name/PI: David Goldhaber-Gordon

Send completed form to: snf-promcommittee@lists.stanford.edu

- The chemical or material. Please provide all common names, trade names, and CAS numbers where appropriate. Include an MSDS, if available; or provide the reason, if not. Make sure to include information for any new secondary chemicals (such as a developer for a new resist). Read the MSDSs as well as the Stanford Chemical Storage Groups to determine the Storage Group Identifier and Main Hazard Class of your chemical/material.

Deposition substrate: thin film (BiSb)2Te3 on a GaAs substrate. Indium backed.

Patterning: In some cases, with PMMA or HMDS+AZ1512.

Carrier wafer: Bare Si or Si + 300nm thermal SiO2 (unpatterned).

- Vendor/manufacturer info: address and phone number, website URL.

Manufactured by an academic group (DRL at UCLA; http://drl.ee.ucla.edu/)

- Reason for request: Please give serious thought to this. If you have any process information (application notes from the vendor, protocol from another lab, experimental methods section of an article), please include it, preferably as attachments. Ask yourself these questions: Is this the latest procedure? Are there newer/safer alternatives that will also work for my project? Will any of the current SNF approved chemicals and materials work for me?

ALD-deposited superconducting NbN has high critical temperatures and fields. We would like to extend previous characterization of ALD deposition of superconducting NbN films in the Fiji3 (reference articles attached) to lower temperatures and plasma powers to allow for processing of fragile materials. We are particularly interested in using such NbN films in heterostrutures with (BiSb)2Te3 family of materials, which generally cannot survive more violent techniques (like sputtering). Both patterned (e-beam or photolitho) and unpatterned ALD is of interest in this context.

- Process Flow: Please provide a detailed process flow description on how and where you proposed to use this chemical. This should include all Lab equipment to be used for processing your wafers once your new chemical or material has been used (even if your new material is a film that is removed, it may still pose potential contamination concerns.) Make sure to include wet benches. Please note that f the chemical/material is to be used in any the "clean" equipment, purity specifications will be needed. This is most important for chemicals/material that are not normally used for VLSI device fabrication. To be allowed into a "clean" tool, the material should MOS grade or better.

- Samples will be cleaned with acetone and IPA outside of SNF.
- When applicable, resist will be spun in SNC, and patterning will be performed by photo- or e-beam lithography in SNC.
- AlOx, HfOx, or NbN ALD films will be deposited in Fiji3. Chips will be placed on a carrier wafer with tweezers that do not touch the inside of the chamber.

- Electrical measurements will be performed in the SNF ExFab Micromanipulator and Lakeshore Hall Probe.

- Further processing and characterization will occur outside of SNF.

- Amount and form. How much will you bring in? Is it solid, powder or liquid? (Note: as a general rule, powders are not permitted in the cleanroom.) Do you need to mix it to use it?

The (BiSb)2Te3 films are solid state and of 8 nm thickness; the GaAs substrates are 3 mm x 3mm with 0.5 mm thickness.

- Storage: Will you be storing your chemical/material at SNF? If so, please note any potential reactivity’s (this should be on the MSDS). Storage groups A, B, D and L are stored in the yellow solvent cabinet in the furnace support area, while storage groups C, E, F and G are stored on top of one of the Pass-through Carts. Ensure your chemical container or material is properly labeled. If there is no available room, it must be stored by in the bulk storage area. You will then need to obtain it from receiving area personnel each time you want to use it and return it to them when you are finished using it (or each time you leave the lab). Note that there is no storage of chemicals/materials in the processing lab or at any wet bench. Not needed.

- Disposal: How will you dispose of any waste or excess chemical or material? In your discussions with experts and vendors, try to determine the best way to dispose of your spent chemicals and by-products. Please refer to the SNF Lab members Safety Manual for the different methods of waste disposal that are available in the lab. Not needed.

To be completed by the PROM COMMITTEE:

Special handling requirements:

On-going approval or staff interaction requirements:

Approved By/Date: