Request Title:  Bi-Sb-Te dry etch in Ox-35 using Methane-H2

Requestor: 	Jim Kruger for Peter Garbuz		Date:  8/13/2019

Non-SNF staff, please complete the following:

Badger ID:jimkruger@yahoo.com

Company name/PI: NMSET/Peter Garbuz

Send completed form to: snf-promcommittee@lists.stanford.edu

- The chemical or material.   Bi-Sb-Te thin film alloys deposited in evaporators outside of SNF, capped with e.g. TiW or other barrier material, mid-process flow. Typical thickness 1 um or less.  5 mm edge relief. (Attached SDS for Bi-Sb-Te materials) Vendor/manufacturer info: peter.garbuz@nmset.com, deposition in a dedicated system offsite.

- Reason for request: Sb and Te are already etched in SNF (PT-mtl) but Bismuth Fluorine and Chlorine compounds are lower volatility.  Literature reports successful etching in Methane-H2 chemistries. Methane, but not H2, is available in PT-mtl.  A working Me-H2 process is available in Ox-35.  The only concern is the introduction of the new materials to Ox-35 and possible residues.  The Sb and Te Methane compounds have high vapor pressures so should not leave residues. Bi has a lower volatility (boiling point = 110C) but should be adequate. We propose to test for  possible residues with test wafers sent out for ICP-MS: 1) after a chamber dry-clean but before any Bi-Sb-Te etching and 2) after Bi-Sb-Te etch followed by chamber dry-clean.   (Attached BoilingPoint spread-sheet and etch paper)

- Process Flow: MEMS structures involving two layers of Bi-Sb-Te and two layers of contact metal (Al-TiW) with sacrificial SiO2 layers.  Wet processing will be avoided.  Only gold-group equipment will be used.  The Bi-Sb-Te layers will be encapsulated by Ti, HfO2 or SiO2 everywhere .  Litho is optical in Heidelberg (in future may migrate to e-beam Raith).  Near the end of the process flow, vapor HF will be used to remove the sacrificial SiO2. (Attached is the detailed process flow)

- Amount and form.  Thin film alloys on wafers, < 1 um thickness, 5 mm edge relief. Estimate ~ 3 wafer batches, 2 Ox-35 etches/batch, ~ 1 batch per 2 weeks.

- Storage:  Only on wafers in the form of thin-films.

- Disposal: No waste other than plasma volatiles out to the scrubbers will be generated.

- To be completed by the PROM COMMITTEE:

Special handling requirements:

On-going approval or staff interaction requirements:

Approved By/Date: