Process or Materials Review Request Form

Request Title: Orthogonal lithography of organic thin-films

Requestor: Quentin Thiburce		Date: 3/28/2019

Non-SNF staff, please complete the following:

Badger ID: thiburce@stanford.edu

Company name/PI: Alberto Salleo

Send completed form to: snf-promcommittee@lists.stanford.edu

- The chemical or material.

Orthogonal photoresist polymer (OSCoR4001, Orthogonal Inc.) with photoacid generator in fluoroether solvent

Causes mild skin irritation; Causes eye irritation; May cause respiratory irritation (MSDS enclosed)

Orthogonal developer, fluoroether + co-solvent (Developer 103a, Orthogonal Inc.)

Causes skin irritation; Causes serious eye irritation; May cause respiratory irritation (MSDS enclosed)

Orthogonal fluoroether stripper (Stripper 900, Orthogonal Inc.)

Causes skin irritation; Causes serious eye irritation; May cause respiratory irritation (MSDS enclosed)

- Vendor/manufacturer info:

##### Orthogonal, Inc.

##### 1999 Lake Avenue

##### Rochester NY 14650

##### 585-254-2775

##### http://www.orthogonalinc.com

- Reason for request:

I am fabricating polymer thin-film transistors and currently have two patterning options for polymer thin-films: a parylene sacrificial layer peel-off method and a germanium hard-mask method. The parylene technique has a limited resolution which is not suitable for my research and the germanium hard-mask is only suitable with water and air-resistant polymers such as PEDOT:PSS, since the germanium hard mask is removed by oxidation in a water bath in which air is bubbled. The semiconducting polymers I need to use are on the other hand highly sensitive to oxygen and water and incompatible with that patterning technique.

I have experience using the commercial orthogonal fluorinated photoresist mentioned above, which was designed to not interact with organic materials. Fluoroethers are used as the solvent, developer and stripper and do not dissolve or degrade sensitive organic polymer thin-films. This is the only process that I know of which fits my patterning requirements. From experience, the OSCoR photoresist is non-sticky and very easy to remove with solvents such as acetone.

Below are links to a selection of relevant research papers:

- https://onlinelibrary.wiley.com/doi/abs/10.1002/adma.200800557

- https://pubs.acs.org/doi/abs/10.1021/ja803493m

- https://onlinelibrary.wiley.com/doi/abs/10.1002/adma.200803291

- https://www.jstage.jst.go.jp/article/photopolymer/22/5/22_5_565/_article

- https://www.spiedigitallibrary.org/conference-proceedings-of-spie/7639/1/Orthogonal-lithography-for-organic-electronics/10.1117/12.848410.full?SSO=1

- https://pubs.rsc.org/en/Content/ArticleLanding/2011/SC/c0sc00612b#!divAbstract

- https://aip.scitation.org/doi/10.1063/1.3650474

- https://pubs.rsc.org/en/Content/ArticleLanding/2016/TC/C5TC03664J#!divAbstract

- Process Flow:

The photoresist will be spin-coated at the Headway spin-coater, soft-baked at 90C for 60 s on the hotplate, exposed in the Heidelberg writer, post-exposure-baked at 90C for 60 s on the hotplate and developed manually at the lithography solvent wet bench. After oxygen plasma etching of the underlying polymer thin-film in the Drytek etcher, the resist will be stripped manually at the lithography solvent wet bench or at the flexible solvent wet bench with the sonicator. I enclosed the manufacturer’s processing guide.

- Amount and form.

Less than 1 litre bottles.

- Storage:

Must be stored away from heat, oxidizing agents, bases and acids. Recommended storage temperature: 4-21C

Possible storage group: G – Not intrinsically reactive, flammable or combustible.

- Disposal:

Liquid waste will be discarded in separate waste containers which may be disposed of with other halogenated waste (local collection).

Solid waste will be discarded in yellow hazardous waste bags with hazardous waste tags and placed in the chemicals pass-through area for pickup.

To be completed by the PROM COMMITTEE:

Special handling requirements:

On-going approval or staff interaction requirements:

Approved By/Date: